OFFER FOR

PROJECT/EQUIPMENT	:	Project

NAME OF PROJECT	:	Heating System (TSM) CLIENT	:	SMS India Pvt. Ltd.                 ENQ. No./DATE	:	Mail enquiry,10/02/2026

OUR REF. /DATE	:	ENCON.04026.122/FBD/JR. R1

DT.28/03/2026

# ENCON Thermal Engineers. (P) Ltd

297, Sector- 21B Faridabad – 121 001 (Haryana) India

Tel: +91(129) 4047847, 2439458

E-mail: sales@encon.co.in	Website: www.encon.co.in

# ENCON Thermal Engineers. (P) Ltd

297, Sector- 21B Faridabad – 121 001 (Haryana) India

Tel: +91(129) 4047847, 2439458,

E-mail: sales@encon.co.in	Website: www.encon.co.in

## ENCON YOUR ANSWER TO THE CONTINUED NEED FOR “ENERGY – CONSERVATION

To,

SMS India Pvt. Ltd.

286, Udyog Vihar-II

Gurugram, Haryana,

Kind Attn. Mr. Rishabh Saxena

Email. rishabh.saxena@sms-group.com

Mob. +91 7895278553

| YOUR REF: | OUR REF: | DATE: |
| --- | --- | --- |
| Mail Enquiry | ENCON.04026.122/FBD/JR. R1 | 28/03/2026 |

Sub: Technical Offer for Tundish Pre-heating Station, Tundish Drying Station, SES Pre-heating Station, Cooling device for tundish.

Dear Sir,

We refer to your mail dated 10/02/2026 and the specifications enclosed herewith, as per the details, we are attaching our Revised technical offer for your kind perusal.

We hope you will find the same in line with your requirements. Should you require any further clarification / information, please feel free to contact us.

Note: DEMBLA make Pneumatic control valve and Shut-off valve is provided in the system.

Thanking you

Regards

Jyotirmay Rabha

ENCON Thermal Engineers Pvt. Ltd.

53 K.M Stone Vill- Baghola, Tehsil-Palwal

Mob +91 8099702129

Email: operations@encon.co.in | www.encon.co.in

TABLE OF CONTENTS

| S No. | Items |
| --- | --- |
| 1. | COMPANY PROFILE` |
| 2. | ABOUT THE CLIENT |
| 3. | TECHNICAL SPECIFICATIONS |
| 4. | GENERAL INFORMATION |

LIST OF ANNEXURES

| Annexure No. |  | Items | Remarks |
| --- | --- | --- | --- |
| Annexure - I |  | SCOPE OF SUPPLY | Attached |
| Annexure - II |  | EXCLUSIONS | Attached |
| Annexure – III |  | PRICE SCHEDULE | Attached |
| Annexure - IV |  | TERMS & CONDITIONS | Attached |

- Company Profile

  - ABOUT US

Founded during the energy crisis of 1978 in India, ENCON Thermal Engineers Pvt Ltd is a pioneer in industrial heating and energy conservation solutions. The name “ENCON” stands for Energy Conservation & Conversion, reflecting the company’s commitment to sustainable energy practices. With over four decades of industry experience, ENCON specializes in providing innovative, reliable, and cost-effective thermal systems and equipment for various industries, including iron and steel, automotive, foundries, aluminum, glass, cement, pharmaceutical, and more.

We are well known for Designing, Manufacturing, Supplying and Commissioning of -FUEL EFFICIENT HEATING SYSTEMS on Turnkey basis

  - INDUSTRIES WE SERVE

  - SOME OF OUR ESTEEMED CLIENTS

  - RESEARCH & DEVELOPMENT

At ENCON, our dedicated R&D division plays a crucial role in driving innovation and new developments. Our team of experts is continuously engaged in research to improve existing products and develop new technologies that enhance energy efficiency and performance. This commitment to R&D allows us to offer cutting-edge solutions that meet the evolving needs of our clients.

Recent developments from our R&D team include advancements in SELF Recuperative Burner and Regenerative Burners, which significantly improve thermal efficiency and reduce energy consumption in high temperature application. We are also exploring new materials and designs to further enhance the durability and effectiveness of our products.

  - OUR COMMITMENT

We have been committed to building long-term partnerships with our clients for the last 46 years and providing tailored solutions that meet specific needs, and contributing to a sustainable future through energy conservation and innovation. ENCON‟s dedication to excellence is reflected in our core values: integrity, quality, customer focus, and continuous improvement.

As a trusted name in the industry, ENCON has successfully executed numerous projects worldwide, earning a reputation for reliability and technical expertise. Our customer-centric approach ensures that we not only meet but exceed client expectations, offering support and guidance throughout the project lifecycle.

  - ENCON’S GLOBAL PRESENCE

### ABOUT THE CLIENT

Client Details:

SMS India Pvt. Ltd. Mr. Rishabh Saxena E. rishabh.saxena@sms-group.com Contact No. +91 7895278553

Marketing Person Details:

Shikha Sharma shikha@encon.co.in Contact No. +91 9871590093

# CASE 1: REFRACTORY WEIGHT CONSIDERED AS 32 TONS

##### TECHNICAL PARAMETER OF TUNDISH PRE-HEATING STATION

| No. Of Burners | 3 Nos. |
| --- | --- |
| Fuel | NG & MG |
| Calorific Value Of NG | 8360 kcal/Nm3 (client confirm) |
| Calorific Value Of MG | 2199 kcal/Nm3 (client confirm) |
| Maximum Temperature | 12000 C |
| Pre-heating Cycle Time | 90 Min. |
| Ignition of Burners | Through NG fired Pilot Burner |
| Lifting & lowering | With Hydraulic cylinder &Power pack |
| Firing Rate for NG | 100 Nm3/hr. per burner |
| Max Fuel Consumptions per cycle | 300 Nm3/Hr |
| Firing Rate for MG | 316 Nm3/hr. per burner |
| Max Fuel Consumptions per cycle | 950 Nm3/Hr |
| Burner Model | Encon dual fuel burner-4A |
| Combustion Air Blower | Centrifugal type |
| Blower Size | 15 HP/32” WC, 2040 Nm3/hr, 1200 CFM |
| Motor for Power Pack | 10 HP |
| Maximum Electrical Load | 18 kW |

- TECHNICAL PARAMETERS TUNDISH DRYING STATION

| No. Of Burners | 3 Nos. |
| --- | --- |
| Fuel | NG & MG |
| Calorific Value Of NG | 8360 kCal/Nm3 (client confirm) |
| Calorific Value Of MG | 2199 kcal/Nm3 (client confirm) |
| Maximum Temperature | 9000 C |
| Pre-heating Cycle Time | 3 Hours |
| Ignition of Burners | Through NG fired Pilot Burner |
| Lifting & lowering | With Hydraulic cylinder &Power pack |
| Firing Rate for NG | 133 Nm3/hr. per burner |
| Max Fuel Consumptions per cycle | 400 Nm3/Hr |
| Firing Rate for MG | 433 Nm3/hr. per burner |
| Max Fuel Consumptions per cycle | 1300 Nm3/Hr |
| Burner Model | Encon dual fuel burner-4A |
| Combustion Air Blower | Centrifugal type |
| Blower Size | 20 HP/32” WC, 2720 Nm3/hr, 1600 CFM |
| Motor for Power Pack | 10 HP |
| Maximum Electrical Load | 22 kW |

SCOPE OF SUPPLY

ANNEXURE –I

SCOPE OF SUPPLY

Our scope of supply will cover design, engineering, manufacturing & supervision for erection, commissioning and performance guarantee.

SUPPORTING STRUCTURE

The necessary steel structural to hold & support the burner holding frame shall be fabricated from mild steel plates and steel sections. Utility piping and other auxiliary equipment’s necessary for regulation of fuel and airflow shall also be fabricated from mild steel plates and steel sections. The heat shield will be provided at the arm and column.

HOOD

The Tundish hood will be fabricated out of mild steel plates of suitable thickness and will be held through suitable supports. Burners will be installed on the hood. The hood will have chimney to vent out the flue gas.

HOOD WITH LINING

The hood would be lined with ceramic fiber blankets having density 128kg/m3. The fiber lining will be held through specially designed SS-304 studs and anchors. Total lining thickness will be 200 mm approx. After lining of the hood, the bulk density of the hood lining shall be 192Kg/m3. The lining will be suitable for the temperature 12000C.

LIFTING & LOWERING MECHANISM OF HOOD

The lifting/lowering of the tundish hood will be done by a hydraulic cylinder in conjunction with hydraulic line from main CCM. The maximum lift of the arm shall be 900. We have considered a control block in our scope which will be fed through the caster hydraulic System.

SAFETY OF LIFTING & LOWERING MECHANISM OF HOOD

An additional mechanical locking arrangement will be provided for preventing the hood to drop down, in case of maintenance of hydraulic cylinder.

ENCON DUAL FUEL BURNER

For preheating the system, we have provided 3 No.  ENCON Dual Fuel Burner to achieve the desired temperature at required time. The burner will consist of the following main components:

- Burner alone

- Micro valve

- CI Burner mounting plate

- Burner Block

- Flexible Hoses Set

- Ball Valve

- Y-Strainer

- Butterfly Valve

# COMBUSTION AIR BLOWER

To supply combustion and atomizing air to the above burner, we will be supplying a centrifugal steel plate air blower of suitable capacity and pressure equipped with squirrel cage induction motor of reputed make such as ABB, or Crompton.

COMBUSTION AIR PIPELINE:

- Pressure gauges with TNV.

- Pressure switch (Low)

- Orifice plate fitted with DPT.

- Pneumatic Flow control valve

- Rotary Joint

NG LINE FOR PILOT BURNER

- Ball valve

- Pressure Gauge with NV

- Pressure Regulator

- Solenoid Valve

- Ball Valve

NITROGEN PURGING

- Ball valve

- Pressure Gauge

- Pressure Regulator

- Pressure Switch High

- Solenoid Valve

- Check Valve

MG LINE FOR MAIN BURNER

The MG line for main burner will be consisting of following main components:

- Butterfly valve

- Pressure gauge with manual cock.

- Pneumatic Shut Off valve

- Pressure Switch Low

- Orifice Plate

- DPT

- Pneumatic Control valve

- Rotary joint

NG LINE FOR MAIN BURNER

The NG line for main burner will be consisting of following main components:

- Ball valve

- Pressure gauge with manual cock.

- Gas filter

- Slam shut-off valve

- Gas pressure regulator

- Safety relief valve

- Pressure Switch Low

- Pressure Switch High

- Solenoid valve

- Orifice Plate

- DPT

- Pneumatic Control valve

PIPELINE

The scope of supply of pipeline shall include combustion air pipeline from blower to the burners along with valves and fittings. Gas pipeline from Gas train to burners. Gas pipeline from T.O.P to the gas train inlet will be in ENCON scope.

INSTRUMENTATION & CONTROL

The burner will be governed by the following scheme fitted on the main gas and air pipeline.

On the main NG pipeline:

Orifice plate with differential pressure transmitter

Pneumatic control valve

Pipeline and pipefittings

On the main MG pipeline:

Orifice plate with differential pressure transmitter

Pneumatic control valve

Pipeline and pipefittings

On the main AIR pipeline:

Orifice plate fitted with differential pressure transmitter

Pneumatic control valve

Pipeline and pipe fittings

TEMPERATURE CONTROL SYSTEM

To control and maintain the temperature of Tundish accurately, the thermocouple will be fitted in the Tundish pre-heater at suitable location. The scheme and sequence of burner operation is described as below:

Temperature control system will consist of the following main component: -

- PLC Siemens S7-1200

- Thermocouple with Temperature transmitter

- Orifice plate fitted with differential pressure transmitter on gas line.

- Orifice plate fitted with differential pressure transmitter on airline.

- Control Valve on the Air & Gas line.

OPERATIONAL SEQUENCE

The temperature of the Tundish will be controlled automatically through PLC. The gas to be used will have to be selected on the control panel.  The thermocouple fitted in the Tundish, will sense the temperature and will give signal to the PLC, the PLC will send a signal to the control valve fitted on the airline, the air control valve will be modulated and accordingly the gas flow be controlled maintaining the mass flow air/gas ratio as the temperature decreases / increases to the set values.

ELECTRICAL CONTROL PANEL

One No. Control panel would be supplied along with the system. Panel box to IP54 protection. The control panel will be fitted with the following main items: -

- Ampere meter & Voltmeter of suitable capacity

- Burner Start / Stop Push Buttons

- Blower Start/Stop

- Emergency Stop Push Buttons

- Selector switches

- Safety, interlocks & alarms

- Indicating lamps and fuses

- PLC Siemens S7 1200 + HMI + Burner Control Unit

- Safety relays and contactor

- MCB of suitable capacity

- Hydraulic cylinder up and down

PAINTING

All steel material will be painted, and surface will be cleaned according to GS 09 provided by the client. Final touch-up painting if required will be done at site after erection.

All bought out components will remain in standard supplier’s finish. Pipelines shall be painted as per color code.

CABLING

All necessary cabling will be done by us to run the system that will include cabling and conducting for power, instrument cables, from control panel to individual equipment. Incoming power supply 415V, 3phase, 50Hz to the control panel will be provided by Client.

- TECHNICAL DATA SES PREHEATING SYSTEM

| Heating medium | Firing through Burner |
| --- | --- |
| No of Burner | 2 |
| Minimum Temperature | Ambient to 12000C |
| Heating Time | 60 minutes |
| Compressed Air Pressure at TOP | 7 Bar. |
| Burner | 90 kW Burner |

OBJECTIVE

Our main objective is to heat the tundish nozzle through routing the burner during the pre-heating of the tundish.

SCOPE OF SUPPLY

Our scope of supply will cover design, engineering, manufacture supply, supervision of commissioning & erection of the following main components:

MANUAL TROLLEY

We shall supply one no. MS fabricated Trolley supported on 4 nos. of wheels (two wheels at one location) with handle arrangement for manual movement of the above SEN Preheater as per the requirement.

BURNER

We are supplying a 90kW burner for the SEN preheater application, designed to meet the required heating load and ensure efficient and uniform preheating performance.

COMPRESSED AIR LINE

# The NG train will consist of the following main components:

- Pressure gauge

- Pressure regulating valve

- Pressure Switch

- Ball valve

NG GAS TRAIN

# The NG train will consist of the following main components:

- Ball valve

- Pressure gauge

- Pressure regulator valve

- Pressure gauge

- Ball valve

- Pressure Switch low

- Pressure Switch high

- Solenoid Valve

ELECTRICAL CONTROL PANEL

One No. Control panel would be supplied along with the system. All the electrical equipment shall be conforming to the Indian Electricity regulations for safety, earthing etc. and panel box to IP54 protection. The control panel will be fitted with the following main items:

- Ampere meter & Voltmeter of suitable capacity

- Indicating lamps and fuses

- MCB, Fues And Connector of Suitable Capacity

PERFORMANCE GUARANTEE PARAMETERS

| EQUIPMENT | HEATING/DRYING TEMPERATURE | HEATING/DRYING TIME |
| --- | --- | --- |
| SES Preheater | Ambient to 12000C | 60 minutes |

- COOLING DEVICE FOR TUNDISH

The cooling device consists of two high capacity cooling fans, each rated at 34,000 Nm3/hr. air flow and 4” WG pressure. The unit is designed for tundish cooling applications and is suitable for handling and positioning via overhead crane.

- Cooling Fan

- Firing Rate: 34000 Nm3/hr with 4” WG

EXCLUSIONS

ANNEXURE –II

All parts and services, which are not specifically mentioned in “Scope of Supply Division”, with particular reference to the following:

EXCLUSIONS FOR ALL SYSTEMS

All parts or materials, which are not specifically mentioned in our Scope of supply with reference to the following.

- All erection and commissioning spares.

- All erection works.

- All Cable, Cabling and earthing materials complete with necessary auxiliaries like junction boxes, cable trays, clamps, etc.

- Civil foundations including civil shuttering drawings, foundation planning, reinforcement details,

- E.O.T. cranes, pulley blocks, lifting tackles and accessories, all tools for erection & commissioning.

PRICE SCHEDULE

ANNEXURE –III

| S. No. | ITEM DESCRIPTION | QTY | UNIT PRICE (INR) | TOTAL PRICE (INR) |  |
| --- | --- | --- | --- | --- | --- |
| 1 | Tundish preheating System | 02 Set | 45,80,000.00 | 91,60,000.00 |  |
| 2 | SES preheating System | 03 Set | 2,30,000.00 | 6,90,000.00 |  |
| 3 | Drying device for Tundish | 01 Set | 50,60,000.00 | 50,60,000.00 |  |
| 4 | Cooling device for Tundish | 01 Set | 7,30,000.00 | 7,30,000.00 |  |
| 5 | Supervision Charges for Erection & Commissioning Erection By Client & Supervision by Encon |  | INR 10500 Per Day (Mechanical) INR 12500 Per Day (PLC & Instrumentation) + To & Fro Air fare from Delhi to Site + Boarding & Loading, Local Conveyance & Medical assistance if required |  |  |
| INR ONE CRORE FIFTY-SIX LAKH FORTY THOUSAND RUPEES ONLY |  |  |  |  | 1,56,40,000.00 |

TERMS & CONDITIONS

ANNEXURE –IV

TERMS AND CONDITIONS

| Prices | : | Ex Works Bhagola Dist: Palwal, Haryana unpacked |
| --- | --- | --- |
| Delivery | : | 12 – 16 Weeks from the date of receipt of PO & Drawing Approval. |
| GST | : | 18% Extra |
| HSN Code | : | 84541000 |
| Pan No. / GST No. | : | AAACE0327M / 06AAACE0327M1ZV |
| Terms of Payment | : | 30 % Advance & balance 70% with taxes & duties against proforma invoice before dispatch. |
| # Packing and forwarding | : | In Client Scope |
| Freight Charges | : | In Client Scope |
| Transit Insurance | : | To be arranged by the client |
| Validity | : | 60 days from the date of our offer. |
| Inspection | : | If required, materials can be inspected at our works before dispatch at your cost with prior intimation. |
| Guarantee | : | Materials shall be guaranteed for a period of 18 Months from the date of dispatch or 12 months from the date of commissioning whichever is earlier Against any manufacturing defects / poor workmanship. |
| Exclusions | : | All parts or materials, which are not specifically mentioned in our Scope of supply & all civil engineering work, Electrical wiring with trench and supports, any fume extraction system, material handling equipment. |