OFFER FOR

PROJECT/EQUIPMENT	:	Project

NAME OF PROJECT	:	Preheaters and Dryers

CLIENT	:	LLYODS METAL & ENERGYLTD

ENQ. No./DATE	:	Enquiry, 20/03/2025

OUR REF. /DATE	:	ENCON.04025.077/FBD/JS/R3

DT.06/05/2025

# ENCON Thermal Engineers. (P) Ltd

297, Sector- 21B Faridabad – 121 001 (Haryana) India

Tel: +91(129) 4047847, 2439458

E-mail: sales@encon.co.in	Website: www.encon.co.in

# ENCON Thermal Engineers. (P) Ltd

297, Sector- 21B Faridabad – 121 001 (Haryana) India

Tel: +91(129) 4047847, 2439458,

E-mail: sales@encon.co.in	Website: www.encon.co.in

## ENCON YOUR ANSWER TO THE CONTINUED NEED FOR “ENERGY – CONSERVATION”

### To,

LLYODS METAL & ENERGY LTD

Kind Attn. Amit K Sangwan (Senior Manager- Projects)

Cont. No.: +91 2269878568

Email: amit.sangwan@lloyds.in

| YOUR REF: | OUR REF: | DATE: |
| --- | --- | --- |
| Mail Enquiry | ENCON.04025.079 /FBD/JS/R3 | 08/05/2025 |

Sub: Offer for Preheaters & Dryers

Dear Sir,

This is with reference to your enquiry about the requirement of preheaters and Driers as per the technical specifications shared dated 20th Mar 2025.

We are enclosing here with our most competitive and comprehensive Revised Offer for the same for your kind perusal.

Though we have tried to make the offer self-explanatory, still if you require any clarification or information, please feel free to contact us.

Regards

### Jagdeep Singh

ENCON Thermal Engineers Pvt. Ltd.

53 K.MStone Vill- Baghola, Tehsil-Palwal Mob +91 9355144580

Email: operations@encon.co.in | www.encon.co.in

NATIONAL ENERGY EFFICIENCY INNOVATION AWARD

## TABLE OF CONTENTS

| S No. | Items |
| --- | --- |
| 1. | COMPANY PROFILE` |
| 2. | ABOUT THE CLIENT |
| 3. | TECHNICAL SPECIFICATIONS |
| 4. | GENERAL INFORMATION |

LIST OF ANNEXURES

| Annexure No. |  | Items | Remarks |
| --- | --- | --- | --- |
| Annexure - I |  | SCOPE OF SUPPLY | Attached |
| Annexure - II |  | EXCLUSIONS | Attached |
| Annexure – III |  | PRICE SCHEDULE | Attached |
| Annexure - IV |  | TERMS & CONDITIONS | Attached |
| Annexure - V |  | REFERENCE LIST OF THE CLIENT | Attached |
| Annexure - VI |  | MAKE LIST | Attached |

- Company Profile

## ABOUT US

Founded during the energy crisis of 1978 in India, ENCON Thermal Engineers Pvt Ltd is a pioneer in industrial heating and energy conservation solutions. The name “ENCON” stands for Energy Conservation & Conversion, reflecting the company’s commitment to sustainable energy practices. With over four decades of industry experience, ENCON specializes in providing innovative, reliable, and cost-effective thermal systems and equipment for various industries, including iron and steel, automotive, foundries, aluminum, glass, cement, pharmaceutical, and more.

We are well known for Designing, Manufacturing, Supplying and Commissioning of -FUEL EFFICIENT HEATING SYSTEMS on Turnkey basis

  - INDUSTRIES WE SERVE

# SOME OF OUR ESTEEMEDCLIENTS

## RESEARCH & DEVELOPMENT

At ENCON, our dedicated R&D division plays a crucial role in driving innovation and new developments. Our team of experts is continuously engaged in research to improve existing products and develop new technologies that enhance energy efficiency and performance. This commitment to R&D allows us to offer cutting-edge solutions that meet the evolving needs of our clients.

Recent developments from our R&D team include advancements in SELF Recuperative Burner and Regenerative Burners, which significantly improve thermal efficiency and reduce energy consumption in high temperature application. We are also exploring new materials and designs to further enhance the durability and effectiveness of our products.

## OUR COMMITMENT

We have been committed to building long-term partnerships with our clients for the last 46 years and providing tailored solutions that meet specific needs, and contributing to a sustainable future through energy conservation and innovation. ENCON‟sdedication to excellence is reflected in our core values: integrity, quality, customer focus, and continuous improvement.

As a trusted name in the industry, ENCON has successfully executed numerous projects worldwide, earning a reputation for reliability and technical expertise. Our customer-centric approach ensures that we not only meet but exceed client expectations, offering support and guidance throughout the project lifecycle.

## ENCON’S GLOBAL PRESENCE

ABOUT THE CLIENT

Client Details:

LLYODS METAL & ENERGY LTD Mr. Amit K Sangwan (Senior Manager- Projects) Contact. No.: +912269878568 Email amit.sangwan@lloyds.in

Marketing Person Details:

Shikha Sharma shikha@encon.co.in Contact No. +91 98715 90093

| Ladle Type | Vertical Ladle Drier Qty - 02 Nos. |  | Horizontal Ladle Preheater – 02 Nos. |  | Vertical Ladle Preheater Qty. – 02 Nos. | Vertical Ladle Drier Qty. – 02 Nos. |
| --- | --- | --- | --- | --- | --- | --- |
| Reference TS | 9991ESP00801-R01 | 9991ESP00801-R01 |  |  | 9991ESP00801-R01 | 9991ESP00801-R01 |
| Ladle Capacity | 50 Ton | 50 Ton |  |  | 50 Ton | 50 Ton |
| Weight of refractory lining | 21.2 Ton | 21.2 Ton |  |  | 21.2 Ton | 21.2 Ton |
| Heating Temperature | Ambient to 1050°C | Ambient to 1050°C |  |  | Ambient to 1050°C | Ambient to 1050°C |
| Heating time | 12 Hours. | 12 Hours. |  |  | 12 Hours. | 12 Hours. |
| Type of fuel | LDO | LDO |  |  | LDO | LDO |
| Calorific value of LDO | LDO: 9000 kcal/ Kg | LDO: 9000 kcal/ Kg |  |  | LDO: 9000 kcal/ Kg | LDO: 9000 kcal/ Kg |
| Firing Rate (R1) | 70 Litre/hr. | 70 Litre/hr. |  |  | 70 Litre/hr. | 70 Litre/hr. |
| Burner Capacity (R1) | LDO – (15 -85) Litre/ hr. | LDO – (15 -85) Litre/ hr. |  |  | LDO – (15 -85) Litre/ hr. | LDO – (15 -85) Litre/ hr. |
| Combustion Air Blower | Centrifugal type, ENCON 40/7.5 | Centrifugal type, ENCON 40/7.5 |  |  | Centrifugal type, ENCON 40/7.5 | Centrifugal type, ENCON 40/7.5 |
| ENCON Burner | Oil Fired ENCON IIP Film Burner | Oil Fired ENCON IIP Film Burner |  |  | Oil Fired ENCON IIP Film Burner | Oil Fired ENCON IIP Film Burner |
| Blower Capacity / Motor rating | 600 CFM@ 40" WC, 7.5 HP | 600 CFM@ 40" WC, 7.5 HP |  |  | 600 CFM@ 40" WC, 7.5 HP | 600 CFM@ 40" WC, 7.5 HP |
| LDO Pumping Unit | Flow Rate: 80 L/hr. | Flow Rate: 80 L/hr. |  |  | Flow Rate: 80 L/hr. | Flow Rate: 80 L/hr. |
| Movement of Hood | Vertical Swiveling through bearing mechanism. | Forward & Reverse trolley drive on rail through Geared motor mechanism. (Rail is excluded in scope) |  |  | Vertical Swiveling through bearing mechanism. | Vertical Swiveling through bearing mechanism. |
| Firing & Ignition of Burner | Automatic Through LPG Fired Pilot Burner | Automatic Through LPG Fired Pilot Burner |  |  | Automatic Through LPG Fired Pilot Burner | Automatic Through LPG Fired Pilot Burner |
| Maximum Electrical Load Required | 18 KW | 18 KW |  |  | 18 KW | 18 KW |

TECHNICAL SPECIFICATIONS OF LADLE PREHEATERS

| Ladle Type | Vertical Hot Metal Ladle Preheater/Dryer Qty – (1+1) Nos. _R3 |
| --- | --- |
| Reference TS | 19991ESP00801/02-R01 |
| Ladle Capacity | 40 Ton |
| Weight of refractory lining | 16 Ton |
| Heating Temperature | Ambient to 1050°C |
| Heating time | 12 Hours. |
| Type of fuel | LDO |
| Calorific value of LDO | LDO: 9000 kcal/ Kg |
| Firing Rate (R1) | 55 Litre/hr. |
| Burner Capacity (R1) | LDO – (15 -85) Litre/ hr. |
| Combustion Air Blower | Centrifugal type, ENCON 40/7.5 |
| ENCON Burner | Oil Fired ENCON IIP Film Burner |
| Blower Capacity / Motor rating | 600 CFM@ 40" WC, 7.5 HP |
| LDO Pumping Unit | Flow Rate: 80 L/hr. |
| Movement of Hood | Vertical Swiveling through bearing mechanism. |
| Firing & Ignition of Burner | Automatic Through LPG Fired Pilot Burner |
| Maximum Electrical Load Required | 18 KW |

# VERTICALLADLE PREHEATERS AND DRYERS

## LADLE HOOD WITH ARM

The Ladle hood will be fabricated out of mild steel plates of suitable thickness and will be held through suitable support. Burner will be installed on the hood. We are providing burner block made of high temperature resistant metal casting for longer and trouble-free life.

## HOOD LINING

Ceramic fiber modular lining of suitable thickness will be provided in the hood. The lining will be held with T– anchors made of SS – 304 specially designed for increase the life of fiber lining & its safety.

## LIFTING & LOWERING MECHANISM

The Lifting - lowering mechanism of the hood will be done by Hydraulic Cylinder & Power-Pack. The maximum lifting and lowering of the Hood will be governed through Limit Switches. An additional mechanical locking arrangement will be provided to prevent the hood from dropping down, in case of failure of the hydraulic system. The hydraulic system will consist of an oil tank, oil pump with motor, suction strainer, pressure gauges with thumb operated needle valve, Direction valves, pilot operated check valves, pressure hoses & level switch, return line filter, all filters with clogging switch and indicator, pressure relieving valve, pressure switch and temperature gauge.

## HYDRAULIC SYSTEM

To take out the Ladle easily after drying/heating, the system will be provided with “Up and Down” movement of hood. The lifting/lowering of the Ladle hood will be provided by a single acting (in push) hydraulic cylinder fitted in column with dedicated power pack. Limit switches LS1 & LS2 will be provided to specify their location. LS1 will indicate its parking position & LS2 will indicate its working position. The maximum lift of the arm should be 85° to its parking position. One no. of dedicated power pack of suitable capacity shall be provided for operating the hydraulic cylinder.

## COMBUSTION AIR BLOWER

To supply combustion air to the above burner, we will be supplying a centrifugal steel plate air blower of suitable capacity equipped with squirrel cage induction motor of reputed make such as ABB/CGL/BB. The blower will be a direct driven type / Impeller mounted on the motor shaft.

## AIR TRAIN FOR MAIN BURNER

The air train will consist of the following main components:

- Pressure gauge with manual cock.

- Orifice Plate with DPT

- Pneumatic Flow control valve

- Butterfly Valve (BFV)

## PILOT BURNER

We shall supply an LPG fired Pilot Burner for automatic ignition of main Burner. The pilot ignition system will be equipped with electrode assembly, UV sensor & ignition transformer & burner control unit. The gas train for the pilot line will consist of Ball Valves, Solenoid Valve & Flexible Hoses.

## LPG LINE FOR PILOT BURNER

We shall be supplying a gas train for Pilot Burner, which will supply the required gas flow and pressure to the pilot burner for ignition of the main burner. LPG pilot line will consist of following main components:

- Ball valves

- Pressure Regulating Valve

- Pressure Gauge

- Solenoid Valve

## ENCON FILM BURNER

For preheating the system, we have provided 1 No. Oil fired ENCON IIP Film Burner of suitable firin g rate to achieve the desired temperature at required time. The burner will be ignited automatically through LPG fired Pilot burner. The burner will consist of the follow ing main components:

- Burner alone

- Micro valve

- High alumina burner block

- CI Burner mounting plate

- Isolation valve on oil line

- Butterfly valves on air line

- Y-Strainer for oil line

## BURNER CONTROL SYSTEM

The above burners will be governed by the following scheme fitted on the main gas and air pipeline:

### On the main oil pipeline:

- Micro Valve

- Flow Meter

- Motorized Control Valve

- Manual Ball Valve

### On the main air pipeline:

- Orifice Plate with DPT

- Pneumatic Flow control valve

- Manual butterfly valve

## TEMPERATURE CONTROL SYSTEM

To control and maintain the temperature of the Ladle accurately, the thermocouple will be fitted in the ladle hood at a suitable location. The temperature control system will consist of the following main component:

- P.L.C (Siemens)-S7 1200

- Thermocouple with temperature transmitter

- Motorized flow control valve on oil line

- Pneumatic flow control valve on Air line

- Orifice Plate with DPT for Airline

- Flow meter in Oil Line

## OPERATIONAL SEQUENCE

The temperature will be controlled automatically through P.L.C The thermocouple fitted in the hood, will sense the temperature and will give signal to the PLC, the PLC will send a signal to the control valve fitted on the airline, the air control valve will be modulated and accordingly the oil flow will be controlled through oil line control valve maintaining the air/Fuel ratio as the temperature decreases / increases to the set values.

## ELECTRICAL CONTROL PANEL

One No. Electrical Control panel would be supplied along with the system. All the electrical equipment shall be conforming to the Indian Electricity regulations for safety, earthing etc. and panel box to IP54 protection.

The control panel will be fitted with the following main items:

- Ampere meter & Voltmeter of suitable capacity

- Combustion Air Blower Start / Stop Push Buttons

- Indicating lamps and fuses

- Safety relays and contactors

- PLC (S7 1200)

- MCB of suitable capacity

- Burner control unit for pilot burner

- Inching bush buttons for hydraulic cylinder operation.

## LDO PUMPING UNIT

To supply fuel oil to the above burner at requisite pressure and temperature, we will supply a suitable pumping unit type Duplex-1 comprising of the following:

- 2 Nos. oil pumps each fitted with suitable electric motor.

- 1 No. each Duplex type coarse and fine filter for the cold and hot oil side respectively.

- 1 No. Pressure regulating valve.

- 1 No. each of Pressure gauge & Temperature gauge.

## PIPELINE

The scope of supply of pipelines shall include combustion air pipeline from blower to the burner and LDO pipeline from LDO pumping unit to burner.

## PAINTING

All steel material will be painted, and the surface will be cleaned according to general specifications of painting provided by the client. All components bought will remain in standard the supplier’s finish. Pipelines shall be painted as per color code.

HORIZONTAL LADLE PREHEATER

## LADLE HOOD WITH ARM

The Ladle hood will be fabricated out of mild steel plates of suitable thickness and will be held through suitable support. Burner will be installed on the hood. We are providing burner block made of high temperature resistant metal casting for longer and trouble-free life.

## HOOD LINING

Ceramic fiber modular lining of suitable thickness will be provided in the hood. The lining will be held with T– anchors made of SS – 304 specially designed for increase the life of fiber lining & its safety.

## HOOD MOVEMENT MECHANISM

Movement of the Ladle hood shall be done through trolley drive & geared motor mechanism. Our scope of supply shall include trolley carrying all the above equipment, and burner firing hood driven by Electro-mechanical drive (Geared Motor). The trolley will be supported on 4 nos. of wheel bearing assembly. The maximum forward and reverse of the trolley will be governed through Limit Switches. The rail for trolley is in CLIENT Scope.

## COMBUSTION AIR BLOWER

To supply combustion air to the above burner, we will be supplying a centrifugal steel plate air blower of suitable capacity equipped with squirrel cage induction motor of reputed make such as ABB/CGL/BB. The blower will be a direct driven type / Impeller mounted on the motor shaft.

## AIR TRAIN FOR MAIN BURNER

The air train will consist of the following main components:

- Pressure gauge with manual cock.

- Orifice Plate with DPT

- Pneumatic Flow control valve

- Butterfly Valve (BFV)

## PILOT BURNER

We shall supply an LPG fired Pilot Burner for automatic ignition of main Burner. The pilot ignition system will be equipped with electrode assembly, UV sensor & ignition transformer & burner control unit. The gas train for the pilot line will consist of Ball Valves, Solenoid Valve & Flexible Hoses.

## LPG LINE FOR PILOT BURNER

We shall be supplying a gas train for Pilot Burner, which will supply the required gas flow and pressure to the pilot burner for ignition of the main burner. LPG pilot line will consist of following main components:

- Ball valves

- Pressure Regulating Valve

- Pressure Gauge

- Solenoid Valve

## ENCON FILM BURNER

For preheating the system, we have provided 1 No. Oil fired ENCON IIP Film Burner of suitable firin g rate to achieve the desired temperature at required time. The burner will be ignited automatically through LPG fired Pilot burner. The burner will consist of the follow ing main components:

- Burner alone

- Micro valve

- High alumina burner block

- CI Burner mounting plate

- Isolation valve on oil line

- Butterfly valves on air line

- Y-Strainer for oil line

## BURNER CONTROL SYSTEM

The above burners will be governed by the following scheme fitted on the main gas and air pipeline:

### On the main oil pipeline:

- Micro Valve

- Flow Meter

- Motorized Control Valve

- Manual Ball Valve

### On the main air pipeline:

- Orifice Plate with DPT

- Pneumatic Flow control valve

- Manual butterfly valve

## TEMPERATURE CONTROL SYSTEM

To control and maintain the temperature of the Ladle accurately, the thermocouple will be fitted in the ladle hood at a suitable location. The temperature control system will consist of the following main component:

- P.L.C (Siemens)-S7 1200

- Thermocouple with temperature transmitter

- Motorized flow control valve on oil line

- Pneumatic flow control valve on Air line

- Orifice Plate with DPT for Airline

- Flow meter in Oil Line

## OPERATIONAL SEQUENCE

The temperature will be controlled automatically through P.L.C The thermocouple fitted in the hood, will sense the temperature and will give signal to the PLC, the PLC will send a signal to the control valve fitted on the airline, the air control valve will be modulated and accordingly the oil flow will be controlled through oil line control valve maintaining the air/Fuel ratio as the temperature decreases / increases to the set values.

## ELECTRICAL CONTROL PANEL

One No. Electrical Control panel would be supplied along with the system. All the electrical equipment shall be conforming to the Indian Electricity regulations for safety, earthing etc. and panel box to IP54 protection.

The control panel will be fitted with the following main items:

- Ampere meter & Voltmeter of suitable capacity

- Combustion Air Blower Start / Stop Push Buttons

- Indicating lamps and fuses

- Safety relays and contactors

- PLC (S7 1200)

- MCB of suitable capacity

- Burner control unit for pilot burner

- Inching bush buttons for hydraulic cylinder operation.

## LDO PUMPING UNIT

To supply fuel oil to the above burner at requisite pressure and temperature, we will supply a suitable pumping unit type Duplex-1 comprising of the following:

- 2 Nos. oil pumps each fitted with suitable electric motor.

- 1 No. each Duplex type coarse and fine filter for the cold and hot oil side respectively.

- 1 No. Pressure regulating valve.

- 1 No. each of Pressure gauge & Temperature gauge.

## PIPELINE

The scope of supply of pipelines shall include combustion air pipeline from blower to the burner and LDO pipeline from LDO pumping unit to burner.

## PAINTING

All steel material will be painted, and the surface will be cleaned according to general specifications of painting provided by the client. All components bought will remain in standard the supplier’s finish. Pipelines shall be painted as per color code.

SCOPE OF SUPPLY

ANNEXURE –I

The Scope of supply will cover design, engineering,	manufacturing,	supply & supervision	for erection & commissioning of the system consisting of the following components.

| Vertical Ladle Driers/Preheaters– 06 Nos. | Horizontal Ladle Preheater – 02 Nos. |
| --- | --- |
| 1. Burner with Burner block & mounting Plate – LDO | 1. Burner with Burner block & mounting Plate – LDO |
| 2. Ladle Hood with Ceramic Fiber lining and Supporting Structure (Column, Arm and Base) | 2. Ladle Hood with Ceramic Fiber lining and Supporting Structure (Trolley and Frame) |
| 3. ENCON IIP Film Burner | 3. ENCON IIP Film Burner |
| 4. Firing & Ignition of the Burner is Automatic through LPG fired Pilot Burner. | 4. Firing & Ignition of the Burner is Automatic through LPG fired Pilot Burner. |
| 5. LDO pumping unit with micro valve. | 5. LDO Pumping unit with micro valve. |
| 6. Hydraulic Cylinder for lifting and Lowering of the system | 6. Forward & Reverse trolley drive on rail and Geared motor mechanism. |
| 7. Combustion Air Blower. | 7. Combustion Air Blower. |
| 8. Temperature Control System with PLC | 8. Temperature Control System with PLC |
| 9. Flow meter and Control valve in Oil and Air line | 9. Flow meter and Control valve in Oil and Air line |
| 10. Interconnecting pipelines from Pumping unit & blower to burner. | 10. Interconnecting pipelines from Pumping unit & blower to burner. |

EXCLUSIONS

ANNEXURE –II

All parts and services, which are not specifically mentioned	in “Scope of Supply Division”,	with reference to the following:

## EXCLUSIONS

All parts or materials, which are not specifically mentioned in our Scope of supply with reference to the following.

- Pipeline & piping from T.O.P to inlet of Pumping unit.

- Civil wok, Cabling, Piping work

- Any fume extraction system

- Any material handling equipment

- Foundation Bolts for column/structure.

- Ladle Stand/Support

PRICE SCHEDULE

ANNEXURE –III

| S. No. | ITEM DESCRIPTION | QTY | UNIT PRICE (INR) | TOTAL PRICE (INR) |  |
| --- | --- | --- | --- | --- | --- |
| 1. | Vertical Ladle Drier – 50 Ton | 02 Set | 17,50,000.00 | 35,00,000.00 |  |
| 2. | Vertical Ladle Preheater – 50 Ton | 02 Set | 17,50,000.00 | 35,00,000.00 |  |
| 3. | Vertical Last Min. Preheater – 50 Ton | 02 Set | 17,50,000.00 | 35,00,000.00 |  |
| 4. | Horizontal Ladle Preheater – 50 Ton | 02 Set | 17,50,000.00 | 35,00,000.00 |  |
| 5. | HM Vertical Ladle Preheater – 40 Ton | 01 Set | 17,50,000.00 | 17,50,000.00 |  |
| 6. | HM Vertical Ladle Dryer – 40 Ton | 01 Set | 17,50,000.00 | 17,50,000.00 |  |
| 7. | Supervision Charges |  | 12,500.00 (Mechanical) 15,000.00 (PLC & Instrumentation) + To & Fro fare from Delhi to Site + Boarding & Loading, Local Conveyance & Medical assistance if required |  |  |
|  | for |  |  |  |  |
|  | Erection |  |  |  |  |
|  | & Commissioning |  |  |  |  |
|  | Erection By Client & |  |  |  |  |
|  | Supervision by Encon |  |  |  |  |
| EARLIER PRICE INR. ONE CRORE NINETY - NINE LAKHS THIRTY-TWO THOUSAND ONLY |  |  |  |  | 1,99,32,000.00 |
| FINAL NO REGRET PRICE: INR. ONE CRORE EIGHTY – FOUR LAKHS THIRTY-SEVEN THOUSAND ONE HUNDRED ONLY |  |  |  |  | 1,84,37,100.00 |
| CLOSING PRICE INR. ONE CRORE SEVENTY FIVE LAKHS ONLY |  |  |  |  | 1,75,00,000.00 |

Note: Based on the final negotiation, we are pleased to offer an additional 7.5% discount, bringing the final price  for the product

| Approx. Weight of the unit System Vertical Ladle Preheater/Dryer | 6.5 | Tonne |
| --- | --- | --- |
| No. of System | 8 | Nos |
| Approx. Weight of the 6 System | 52 | Tonne |

| Approx. Weight of the Unit System Horizontal Ladle preheater | 5 | Tonne |
| --- | --- | --- |
| No. of System | 2 | Nos |
| Approx. Weight of the 6 System | 10 | Tonne |

TERMS & CONDITIONS

ANNEXURE –IV

PRICES	: Ex Works Bhagola, Dist: Palwal, Haryana unpacked

DELIVERY	: 10 - 12 Weeks from our ENCON works from the date of receipt of

Advance & Drawing approval whichever is later

GST	: 18 % Extra

HSN CODE	: 84541000

PAN NO / GST NO	: AAACE0327M / 06AAACE0327M1ZV

TERMS OF PAYMENT	: 30 % advance with the purchase order

70 % against Performa invoice prior to dispatch

PACKING &

FORWARDING	: 4% & 2 % respectively FREIGHT CHARGES	: In client’s Scope TRANSIT INSURANCE	: To be arranged by the client

VALIDITY	: 45 days from the date of our offer

INSPECTION	: If required, materials can be inspected at our works before

dispatch at your cost with earlier intimation.

GUARANTEE	: Materials will be guaranteed for a period of 18 months from the date

of dispatch or 12 months from the date of commissioning whichever is earlier against any manufacturing defects / poor workmanship

.

REFERENCE LIST

ANNEXURE –V

| REFERENCE FOR LADLE PREHEATERS |  |  |  |
| --- | --- | --- | --- |
| 1 | JSPL, Raigarh | Vertical Ladle Preheater - 65T |  |
| 2 | Vardhman Special Steel Ltd. | 30T Vertical Ladle Preheater - NG |  |
| 3 | RML Chaliyama | Vertical Ladle Dryer & Horizontal Ladle Preheater |  |
| 4 | Ganesh Steel & Alloys Ltd. | 10T Vertical Ladle Preheater |  |
| 5 | SMS India Pvt. Ltd. | Ladle Stirring Valve Rack |  |
| 6 | Emirates Steel | Vertical Ladle Preheater - 50T |  |
| 7 | KISCO CASTINGS INDIA LIMITED | Ladle Preheaters services |  |
| 8 | RML Dhenkanal | Vertical Ladle Dryer & Horizontal Ladle Preheater |  |
| 9 | BHEL, Haridwar | Gas Fired Vertical Ladle Preheaters - 50 Ton |  |
| 10 | Rungta Mines Limited KSP | Vertical Ladle Drier and Horizontal LPH |  |
| 11 | JSW Dolvi | Horizontal & Vertical Ladle Preheaters |  |
| 12 | JSL Jajpur | Horizontal & Vertical Ladle Preheaters |  |
| 13 | JSW, Vidyanagar, Bellary | Vertical Ladle PH - 165T (Mechanical Only) |  |
| 14 | SMS India Pvt. Ltd. | Ladle Preheaters 350T & 370T |  |
| 15 | Aarti Steels Limited | Ladle | Hood & Heater - 4kW |
| 16 | Glorious Green Gate Co.Ltd | 20 Vertical Ladle Preheaters |  |
| 17 | Devbhoomi Casting Pvt. Ltd. | 15 T Ladle Preheaters |  |
| 18 | RML Chaliyama | Vertical Ladle Dryer & Horizontal Ladle Preheater |  |
| 19 | Muscat Steel | Ladle | Pre Heaters |
| 20 | Electrotherm | Ladle | Pre Heaters |
| 21 | Mukand Steel | Ladle | Pre Heater |
| 22 | Danieli India | Ladle | Pre Heaters |
| 23 | Primetals | Ladle | Pre Heaters |
| 24 | JSW Salem | Ladle | Pre Heater |
| 25 | BSRM | Ladle | Pre Heater |

MAKE LIST

ANNEXURE –VI

| S. No. | ITEM | MAKE |
| --- | --- | --- |
| 1. | MS Structure | JINDAL |
| 2. | Combustion Equipment: |  |
|  | a. Encon Film Burner | ENCON |
|  | b. Blower | ENCON |
|  | c. Pumping Unit | ENCON |
| 3. | Ceramic fiber | MURUGAPPA |
| 4. | Control Panel | ENCON |
| 5. | Ball Valve | L&T/LEADER |
| 6. | Butterfly valve | L&T/LEADER |
| 7. | Hydraulic Cylinder | VARITECH |
| 8. | Gearbox | POWERTEK |
| 9. | Pilot Burner | ENCON |
| 10. | UV Sensor | LINEAR |
| 11. | Burner Controller Unit | LINEAR |
| 12. | Ignition Transformer | COFI/DANFOSS |
| 13. | LPG Pilot Gas Train | MADAS |
| 14. | Control Valve | CAIR/PNEUTORK |